# Tailgate Safety Meeting #3

# Trenching & Excavation Hazards (OSHA Compliant)

Duration: 10 minutes

Audience: All field crew and subcontractors

Objective: To recognize, prevent, and respond to trenching and excavation hazards.

## Script to Read Aloud (5–7 minutes)

Foreman Reads:

Good morning crew. Today we’re going to talk about trenching and excavation — one of the most dangerous operations we perform on a construction site. Cave-ins are the leading cause of fatalities in trenching. Even a shallow trench can collapse and bury a worker in seconds.

By OSHA definition, any cut in the ground that is deeper than it is wide is considered an excavation. If it's deeper than 5 feet, it requires a protective system unless it's made entirely in stable rock. That means sloping, benching, shoring, or using a trench box.

Never enter a trench unless it has been properly inspected by a competent person. That person must check for signs of collapse, water infiltration, or other hazards like adjacent structures or heavy equipment traffic nearby.

Keep spoils and materials at least 2 feet back from the edge. Use ladders or safe access points every 25 feet. And never jump into or out of a trench. Use designated entry and exit points.

👉 DEMO SUGGESTION: If you're working near an active trench, walk the crew over to the site and review spoil pile distance, access points, and any shoring or boxes being used.

## Main Bullet Points (for Foreman)

Any trench over 5 feet deep must have a protective system.

Never enter a trench unless inspected by a competent person.

Keep spoil piles at least 2 feet from the trench edge.

Use ladders every 25 feet for safe access and egress.

Watch for signs of water, collapse, or instability.

Use trench boxes, shoring, or sloping systems properly.

If near an active trench, demonstrate proper setup and access rules.

## Engagement Tip

- Ask: “Has anyone here witnessed a trench collapse or close call?”
- Walk the crew to a trench and inspect conditions together.
- Reinforce: ‘No trench is safe to enter unless it’s been inspected today.’