# Tailgate Safety Meeting #15

# Confined Space Awareness (OSHA Compliant)

Duration: 10 minutes

Audience: All field crew and subcontractors

Objective: To recognize, assess, and control hazards associated with confined space entry on the job site.

## Script to Read Aloud (5–7 minutes)

Foreman Reads:

Good morning crew. Today we’re covering confined space awareness — a critical topic because these areas can be extremely hazardous, and the risks are often invisible until it's too late.

A confined space is any area that:
- Has limited entry or exit
- Is not designed for continuous occupancy
- May have a hazardous atmosphere

Examples include utility vaults, tanks, crawl spaces, sewers, manholes, and large ducts. These spaces can quickly become deadly due to lack of oxygen, toxic gases, engulfment hazards, or poor visibility.

Never enter a confined space unless:
- It has been tested for oxygen, flammable gases, and toxic vapors
- You are trained and authorized
- You have a permit (if required) and the proper PPE
- There is an attendant outside and a rescue plan in place

👉 DEMO SUGGESTION: Identify a confined space on the site. Review atmospheric testing procedures and entry requirements with your crew.

## Main Bullet Points (for Foreman)

Confined spaces can be deadly — even if they appear safe.

Test the air before entry: oxygen, flammables, and toxins.

Follow the permit-required confined space entry protocol.

Ensure ventilation, PPE, and monitoring systems are in place.

Have an attendant present at all times during entry.

Never enter a confined space alone or untrained.

Demonstrate how to identify and secure confined spaces on site.

## Engagement Tip

- Ask: “Has anyone worked in or around a confined space without realizing it?”
- Locate a confined space on site and review the entry checklist.
- Reinforce: ‘If you’re not trained and authorized — don’t go in.’