# Tailgate Safety Meeting #18

# Trenching & Excavation Safety (OSHA Compliant)

Duration: 10 minutes

Audience: All field crew, excavation teams, and subcontractors

Objective: To understand the hazards of trenching and excavation and the protective measures required to prevent cave-ins, falls, and utility strikes.

## Script to Read Aloud (5–7 minutes)

Foreman Reads:

Good morning crew. Today’s safety meeting is about trenching and excavation safety. Excavation work is one of the most dangerous tasks in construction. Cave-ins, falls, falling tools, and underground utilities can all pose serious hazards.

By OSHA’s definition, a trench is any narrow excavation deeper than it is wide, no more than 15 feet wide at the bottom. A cave-in can bury a worker in seconds — one cubic yard of soil can weigh over 3,000 pounds.

Key trench safety requirements:
- A **competent person** must inspect trenches daily and after any weather or activity that may affect stability.
- Protective systems (sloping, shoring, or shielding) must be used in trenches deeper than 5 feet.
- Keep spoils and materials at least 2 feet from the trench edge.
- Use ladders or egress within 25 feet of all workers.
- Always locate and mark underground utilities before digging.

👉 DEMO SUGGESTION: Walk to an open trench or excavation and identify if proper sloping, shoring, or shielding is in place. Review escape ladder placement and spoil location.

## Main Bullet Points (for Foreman)

Trenching is one of the most hazardous operations — cave-ins kill workers every year.

Always use sloping, shoring, or trench boxes for trenches over 5 feet deep.

Inspections must be done daily by a competent person.

Keep heavy equipment and spoils away from the edge of the trench.

Install access ladders every 25 feet in trenches 4 feet or deeper.

Call 811 before digging — always locate and mark utilities.

Never enter an unprotected trench — it's not worth your life.

## Engagement Tip

- Ask: “Have you ever seen or been in a trench that felt unsafe?”
- Have the crew inspect the closest excavation together.
- Reinforce: ‘If it’s not safe, stay out — speak up.’