# Tailgate Safety Meeting #1

# Fall Protection & Ladder Safety (OSHA Compliant)

Duration: 10 minutes

Audience: All field crew and subcontractors

Objective: To prevent falls, the #1 cause of fatalities in construction

## SCRIPT TO READ ALOUD (5–7 minutes)

Good morning crew. Today we’re talking about something that kills more workers than anything else on site — falls. Falls from ladders, scaffolds, rooftops, or even from a few feet up can lead to serious injuries or death. OSHA requires us to protect ourselves anytime we’re working at heights of 6 feet or more in construction.

Let’s start with fall protection. That means using guardrails, safety nets, or personal fall arrest systems (PFAS) — and using them correctly.

A full-body harness connected to an anchor point won’t help if it’s not worn right or if the anchor fails. So if you’re tied off, ask yourself — where is my anchor? Is it rated for 5,000 lbs.? Am I connected properly with a locking snap hook? Are there any frays or cuts in my lanyard?

👉 If you have a harness available on-site, have each employee take a moment to practice putting it on and off. Or ask someone familiar with proper usage to demonstrate correct harness adjustment and tie-off procedure.

Now — let’s talk ladders. Everyone thinks they know how to use one, but most ladder injuries come from bad habits. Like standing on the top rung, using a ladder on uneven ground, or carrying tools while climbing.

Here are a few reminders:
- Always face the ladder when climbing.
- Keep three points of contact — two hands and one foot, or two feet and one hand.
- Never use a ladder in front of a door without locking or blocking the door.
- Don’t overreach — reposition the ladder.
- Make sure it’s the right ladder for the job — right height, right type.

👉 If there is a ladder available, have someone demonstrate proper climbing technique with three points of contact, and explain common misuse scenarios.

Lastly, inspect your ladders. If it’s bent, cracked, missing a foot pad, or covered in paint and glue — tag it and pull it from service.

Falls can be prevented with planning, training, and proper equipment. If you see an unsafe ladder, or someone not tied off — speak up. Protect yourself and your team.

## MAIN BULLET POINTS (FOR FOREMAN)

Falls are the #1 cause of death in construction.

Use fall protection at 6 feet or higher (guardrails, PFAS, safety nets).

Anchor points must hold 5,000 lbs. or be engineered.

Inspect your harness, lanyard, and connectors before each use.

Use ladders correctly:

- 3 points of contact

- Never top step

- Stable base

- Don’t carry tools up/down

Inspect ladders regularly and tag damaged ones out of service.

If tools or equipment are discussed (e.g., harnesses, ladders), include live demonstration or hands-on use.

Speak up — fall protection is everyone’s job.

## ENGAGEMENT TIP

Ask:
- “What’s the last unsafe ladder you saw on site?”
- “Who can show the correct way to tie off a harness right now?”