# Tailgate Safety Meeting #41

# Underground Utility Line Locating & Marking (OSHA Compliant)

Duration: 10 minutes

Audience: Utility crews, excavation workers, machine operators, and site supervisors

Objective: To ensure safe excavation through proper utility locating, marking, and communication with utility companies.

## Script to Read Aloud (5–7 minutes)

Foreman Reads:

Before any digging, boring, trenching, or grading begins — you must know what’s under your feet. Underground utilities pose serious risks of explosion, electrocution, flooding, and gas leaks.

Here's how we stay safe:
- Always call 811 or your state’s utility notification center before digging — it’s the law.
- Wait for all utility owners to mark their lines — NEVER assume a site is clear.
- Respect the tolerance zone — typically 18–24 inches on either side of a marked line.
- Hand dig or vacuum excavate when working near marked utilities.
- Use proper color codes for markings:
   • Red = Electric
   • Yellow = Gas/Oil/Steam
   • Orange = Communication
   • Blue = Potable Water
   • Green = Sewer/Drain
   • White = Proposed excavation
- Maintain updated utility maps and keep markings visible.
- Immediately stop work and notify supervisors if you expose or damage any line.
- Treat all exposed utilities as live and energized — don’t assume they’re inactive.

👉 DEMO SUGGESTION: Walk the crew to a marked area on-site and review each color and its meaning. Practice hand digging near a mock utility mark.

## Main Bullet Points (for Foreman)

Call 811 before you dig — every time.

Wait for all utility markings before starting excavation.

Know and respect utility color codes and tolerance zones.

Use non-destructive methods when digging near utilities.

Never assume a utility is inactive — always treat as live.

Stop work immediately if a utility is exposed or damaged.

Keep utility maps updated and accessible on-site.

Train your team on how to identify and avoid underground hazards.

## Engagement Tip

- Ask: “Have you ever hit or come close to hitting an underground line?”
- Let someone describe what steps they took before digging.
- Reinforce: ‘Every mark on the ground could save a life — or cost one if ignored.’