# Tailgate Safety Meeting #42

# Trench Collapse Prevention & Shoring Systems (OSHA Compliant)

Duration: 10–12 minutes

Audience: Excavation crews, equipment operators, site supervisors, utility workers

Objective: Prevent serious injury or death from trench collapses by implementing proper shoring, shielding, and sloping practices.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Trench work is one of the most dangerous activities in construction. A cubic yard of soil can weigh as much as a car — and if a trench collapses, there’s little to no time to escape. Most fatalities occur in trenches less than 10 feet deep.

Our safety depends on prevention, not rescue.

Follow these trench safety principles:
- OSHA requires protective systems for trenches 5 feet deep or more unless the excavation is made entirely in stable rock.
- Protective systems include:
   • Sloping: cutting trench walls at an angle away from the excavation.
   • Shoring: using hydraulic jacks and timber or aluminum systems to support trench walls.
   • Shielding: trench boxes that protect workers if a collapse occurs.
- A Competent Person must inspect the trench and protective systems daily and as conditions change (weather, water accumulation, vibration).
- Never enter an unprotected trench — even for “just a second.”
- Trenches deeper than 4 feet must have safe egress like ladders or ramps within 25 feet of all workers.
- Keep heavy equipment and materials at least 2 feet from the edge.
- Watch for changes in soil condition, standing water, or cracks along the trench wall.
- Stay alert to vibration from machinery, traffic, or trains.
- Never assume a trench box is safe unless inspected.

👉 DEMO SUGGESTION: If safe to do so, walk the crew to a current excavation and inspect it together. Identify the type of protection used and discuss potential red flags.

## Main Bullet Points (for Foreman)

Use sloping, shoring, or shielding in trenches 5 feet deep or more.

Have a Competent Person inspect trenches daily or after any changes.

Ensure ladders or ramps are placed every 25 feet in trenches over 4 feet deep.

Never work in an unprotected trench — even for a moment.

Keep spoils and equipment at least 2 feet from trench edges.

Watch for soil movement, cracks, standing water, or vibrations.

Use trench boxes correctly — never modify or climb on them.

Report any signs of trench instability immediately.

Ensure all workers understand and recognize the protective systems in use.

## Engagement Tip

- Ask: “What signs of collapse have you seen before?”
- Let workers point out how they would exit a trench on-site.
- Reinforce: ‘You don’t get a second chance when a trench caves in — protect yourself every time.’