# Tailgate Safety Meeting #44

# Confined Space Entry for Utility Workers (OSHA Compliant)

Duration: 10–12 minutes

Audience: Utility crews, plumbers, electricians, foremen, and safety supervisors

Objective: Ensure proper identification, permitting, and safety procedures for entering and working in confined spaces.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Confined spaces can be deadly. Many workers don’t realize how fast air quality, toxic gases, or engulfment hazards can kill inside a manhole, vault, or tank. You may only get seconds to act — and rescue is often impossible once things go wrong.

A confined space is:
- Large enough for a worker to enter,
- Has limited entry or exit,
- Is not designed for continuous occupancy.

Examples include manholes, vaults, storm drains, tanks, and underground pump stations.

Many of these become *permit-required confined spaces* when they have one or more of the following:
- Hazardous atmosphere (e.g. low oxygen, methane, hydrogen sulfide),
- Risk of engulfment,
- Dangerous internal configuration,
- Any other serious safety hazard.

OSHA requires:
- A Confined Space Entry Permit system,
- Pre-entry atmospheric testing,
- Ventilation before and during entry,
- Standby attendants trained in rescue procedures,
- Lockout/tagout of all energy sources.

No one may enter a confined space unless they are properly trained and authorized, the atmosphere is confirmed safe, and all safety measures are in place.

👉 DEMO SUGGESTION: If you have gas detection equipment, show how to check oxygen, CO, H2S, and LEL levels. Or have a worker walk through a sample permit process from your company.

## Main Bullet Points (for Foreman)

Always identify and evaluate any confined space before entry.

Use a valid permit system when required — and follow it step by step.

Test for atmospheric hazards before entry using calibrated detectors.

Ventilate the space before and during entry if needed.

Ensure a trained attendant is present at all times — never leave the hole unattended.

Use communication methods (radios, ropes, etc.) to stay in contact with the entrant.

Have retrieval systems and rescue plans ready before entry.

Lockout all energy sources (pumps, valves, fans) before entry.

Only authorized and trained personnel may enter permit-required confined spaces.

## Engagement Tip

- Ask: “What are some confined spaces you’ve worked in — and what risks did you see?”
- If available, demonstrate gas detector use and explain safe oxygen and LEL ranges.
- Reinforce: ‘Confined spaces kill workers every year — the rules exist for a reason.’