# Tailgate Safety Meeting #46

# Vacuum Excavation Safety (OSHA Compliant)

Duration: 10–12 minutes

Audience: Utility and excavation crews, vac-truck operators, safety managers

Objective: Prevent injuries and utility damage by using vacuum excavation safely and correctly.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Vacuum excavation (also known as soft digging or potholing) uses high-pressure air or water to loosen soil, which is then removed by a vacuum. It’s one of the safest ways to expose underground utilities — but only if done right.

Improper vac-ex practices can still cause injury or damage:
- Pressurized water can cut into cables or fiber optics.
- Improper grounding can result in electric shock.
- Poor traffic control around vac trucks can create roadside hazards.
- Inhalation of dust or contact with sewage lines can cause exposure illnesses.

Key safety protocols include:
- Always locate utilities via 811 and site markings before beginning.
- Use the lowest effective pressure for water excavation to avoid utility damage.
- Maintain safe distances from marked lines — always hand dig the last 18–24 inches.
- Wear eye protection, gloves, and respiratory protection when necessary.
- Use grounding rods when working near electric lines.
- Keep hoses in good condition — check for leaks and pressure problems.
- Maintain clear sightlines and communication between operator and ground crew.

👉 DEMO SUGGESTION: Review pressure settings for both air and water vacs, and demonstrate proper wand technique if safe to do on-site.

## Main Bullet Points (for Foreman)

Verify utility locations and mark bore paths before vacuuming.

Use proper PPE including gloves, safety glasses, hearing protection, and respirators where needed.

Operate at the lowest effective water pressure to avoid utility damage.

Keep vacuum hoses away from energized lines — use grounding equipment.

Ensure traffic control is established around vac trucks in roadways.

Inspect equipment before use: hoses, valves, tank seals, pressure gauges.

Use proper trench or hole covers when leaving an excavation unattended.

Watch for signs of hazardous materials or contaminated soil.

Do not allow untrained workers to operate the vac wand or pressurized system.

## Engagement Tip

- Ask: “Has anyone seen damage caused by improper vac work?”
- If time allows, walk over and evaluate the vac setup on the truck — talk about common mistakes.