# Tailgate Safety Meeting #48

# Equipment Spotter Communication Protocols (OSHA Compliant)

Duration: 10–12 minutes

Audience: Equipment operators, spotters, laborers, site foremen

Objective: Prevent struck-by incidents and improve coordination between equipment operators and spotters.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Construction sites are loud, fast-moving, and full of blind spots. Spotters are essential to preventing accidents and keeping workers out of harm’s way. But being a spotter isn’t just about waving your hands — it’s about clear, consistent communication.

The risks we’re guarding against:
- Workers getting run over or pinned by heavy equipment.
- Equipment damaging structures, materials, or utilities.
- Confusion between operator and spotter signals.
- Distractions or poor visibility leading to strikes.

Spotter Safety Protocols:
- Spotter must be visible with high-vis vest and in operator’s line of sight.
- Establish hand signal agreement before starting — no improvising.
- Use radios if equipment is large or visibility is obstructed.
- Maintain eye contact or continuous communication.
- Never walk backward while spotting. Always face equipment.
- Only one spotter per piece of equipment — avoid conflicting instructions.
- Operators must STOP if spotter is not visible or unclear.

👉 DEMO SUGGESTION: Practice 5–6 standard hand signals (stop, back, left, right, boom up/down, emergency stop) and quiz crew members.

## Main Bullet Points (for Foreman)

Review and practice standardized hand signals before operation.

Spotters should wear reflective vests and remain alert and visible.

Use two-way radios for noisy or obstructed environments.

Only one spotter per machine — too many creates confusion.

Spotter should never assume operator sees them without confirmation.

Operators must stop if signal is unclear or spotter disappears from view.

No distractions — no phones or side conversations for either party.

Position spotters where they can clearly see both the path and operator.

Document communication plan during daily pre-task meetings when possible.

## Engagement Tip

- Ask: “Who here has acted as a spotter before? What went right — or wrong?”
- Demonstrate poor vs. clear signals and discuss how confusion can escalate quickly.