# Tailgate Safety Meeting #49

# High-Rise Fall Object Control & Toe-Board Use (OSHA Compliant)

Duration: 10–12 minutes

Audience: Structural workers, ironworkers, scaffold users, safety leads

Objective: Prevent injuries from falling tools, materials, and equipment during high-rise work.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Working on high-rises presents unique risks — and one of the biggest is falling objects. A small tool dropped from 10 stories up becomes deadly. A wrench, a bolt, or even a tape measure can kill someone below. This is why object control at height is not optional — it’s essential.

Main Risks:
- Tools, bolts, or material dropped from scaffolds, edges, or tool belts.
- Lack of toe-boards or edge barriers.
- Poorly secured loads on scaffolds or platforms.
- Workers passing under active high-rise areas without overhead protection.
- Openings in decking or perimeter edges.

Prevention Strategies:
- Install toe-boards on all elevated platforms, scaffolds, and work areas.
- Use tool lanyards or tethered tools when working at height.
- Avoid stacking material near unprotected edges.
- Cover or barricade floor openings immediately.
- Wear hard hats at all times below overhead activity.
- Use netting, debris panels, or barricades to shield pedestrian paths or entry points.
- Communicate drop zones and enforce “no-walk” areas below hoisting operations.
- Secure all materials, especially in windy conditions.

👉 DEMO SUGGESTION: Show examples of tool tethers and demonstrate how they attach to common tools. If working on scaffolding or near decking, inspect toe-board installation.

## Main Bullet Points (for Foreman)

Install toe-boards along all scaffolding, elevated decks, and leading edges.

Use tool lanyards when working 6 feet or more above another worker or path.

Mark and barricade drop zones — especially below active edges or hoist points.

Hard hats are mandatory near any high-rise activity — even for short exposure.

Secure materials on windy days and avoid overloading platforms.

Immediately repair or cover floor holes/openings in elevated decks.

Use netting or debris containment panels where toe-boards are not feasible.

Maintain awareness — never toss tools or materials to coworkers above/below.

Conduct overhead inspections before work begins and periodically throughout the day.

## Engagement Tip

- Ask: “Has anyone ever witnessed a falling object close call? What was the outcome?”
- Optional: Walk around the site and check for missing toe-boards or unsecured materials.