# Tailgate Safety Meeting #50

# Structural Steel Erection and High-Rise Rigging Safety (OSHA Compliant)

Duration: 10–15 minutes

Audience: Ironworkers, riggers, crane operators, structural crews

Objective: Prevent falls, crane hazards, and rigging failures during steel erection and rigging operations at heights.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Steel erection is dangerous work, especially on high-rise projects. Falls, collapses, and struck-by hazards are real threats, so every worker must be trained and dialed in. Mistakes at height can mean serious injury or death — and it can happen fast.

Key Risk Areas:
- Improper rigging or overloaded picks.
- Unstable steel components before they are secured.
- Falls from open decking or unprotected edges.
- Crane miscommunication or swing hazards.
- Weather — wind can destabilize loads or unbalanced members.

Safety Protocols for Steel Erection:
- All workers must use 100% tie-off fall protection — lifelines or harnesses — when exposed to falls over 6 feet.
- Use connectors with lanyards designed for steel erection work — no standard residential gear.
- Inspect rigging and shackles before each lift. If unsure, tag it out.
- Never stand under suspended loads — communicate drop zones and stay clear.
- Install perimeter cables and guardrails on open floors ASAP.
- Always set steel with a signalman — don’t rely on hand gestures alone at high rise.
- Watch the weather — delay lifts during strong winds.
- Bolt-up crew must not proceed until members are plumbed, braced, and secure.
- Lay down decking with caution — never “walk the beam” unprotected.

👉 DEMO SUGGESTION: Review common rigging hardware (shackles, chokers, chains) and have crew practice safe load securing or walk through a sample lift plan with operator input.

## Main Bullet Points (for Foreman)

Use qualified riggers and crane operators for all structural lifts.

Tag out damaged rigging gear and inspect before every shift.

Ensure tie-off points and lifelines are in place before decking begins.

Maintain communication between signal person and operator at all times.

Never walk under a load — set clear exclusion zones.

Stop work if wind, lightning, or visibility threatens the lift.

Install handrails and cables as soon as possible on open decks.

Check beam connections before climbing or loading weight onto them.

Use tag lines to control load swing — especially on multi-story lifts.

## Engagement Tip

- Ask: “Have you ever seen a near miss with a suspended steel load or unprotected edge?”
- Optional: Walk through yesterday’s rigging work — ask crew what went well and what could be improved.