# Tailgate Safety Meeting #52

# Wind Hazards and Weather-Triggered Shutdown Protocols (OSHA Compliant)

Duration: 10–15 minutes

Audience: All construction trades working outdoors or at heights

Objective: Identify wind-related risks and establish clear stop-work procedures tied to weather conditions.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

High winds and sudden weather changes are serious threats on construction sites, especially on high-rise buildings, roof work, crane operations, and scaffold setups. Workers have been blown off roofs, loads have swung unpredictably, and unsecured materials have become deadly projectiles.

Key Wind Hazards:
- Scaffold instability or toppling in wind gusts.
- Suspended loads swinging during crane lifts.
- Roofing materials or tools blowing off elevated surfaces.
- Aerial lift platforms tipping over or drifting.
- Worker balance compromised on open edges, decking, or roofs.

Weather-Triggered Shutdown Protocols:
- Monitor weather daily — assign someone to check wind forecasts hourly when high winds are expected.
- Have a written wind threshold policy. For example:
    • Stop crane lifts over 20–25 mph wind speeds (manufacturer limits may be lower).
    • Cease roof work or elevated installs over 30 mph.
    • Take down scaffolding wrap or tarps at 25+ mph.
- Train crews on these thresholds — don’t rely on ‘feel’ alone.
- Remove unsecured materials and tools when wind gusts begin.
- Pause work and shelter crews when lightning or extreme weather hits.
- Use wind-rated tie-off systems and extra anchor points where needed.

👉 DEMO SUGGESTION: Use a handheld anemometer to show real-time wind speed. Review jobsite’s current stop-work thresholds and discuss what each trade should do when they’re hit.

## Main Bullet Points (for Foreman)

Identify wind-sensitive tasks — cranes, lifts, roofing, scaffold, sheeting.

Set clear stop-work wind thresholds by task and follow them.

Check and document wind conditions — use a wind gauge if needed.

Secure all materials and tools at height when winds pick up.

Communicate to crews who has authority to call a shutdown.

Lightning, hail, or storms? Suspend outdoor work immediately.

Tie-off anchor systems must be rated for windy conditions.

Wind can vary by floor level — check rooftop wind specifically.

## Engagement Tip

- Ask: “What’s the strongest wind you’ve worked in — and was it safe?”
- Optional: Roleplay calling a wind-related shutdown — what do crews do next?