# Tailgate Safety Meeting #54

# Vertical Material Hoist Safety and Overload Prevention (OSHA Compliant)

Duration: 10–15 minutes

Audience: Hoist operators, laborers, superintendents, and structural crews on high-rise projects

Objective: Reduce accidents and mechanical failures related to vertical hoisting systems and overload risks.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Material hoists are used on high-rise and large-scale commercial construction sites to move tools, supplies, and debris up and down. If operated improperly or overloaded, hoists can fail catastrophically, causing serious injury, loss of load, or collapse.

Common Hazards:
- Exceeding rated load capacity of hoist platform or motor
- Unbalanced loads or improperly stacked materials
- Inadequate guarding or fencing around hoist shaft
- Workers riding material hoists not designed for personnel
- Untrained operators or failure to inspect equipment daily

Best Practices for Safe Material Hoist Use:
- Know the load rating of each hoist — never exceed the limit.
- Secure and balance the load; never let items hang over edges.
- Only trained, authorized personnel may operate hoists.
- Ensure gates and fencing are closed before moving the platform.
- Perform a visual inspection before each shift: cables, brakes, switches.
- Post maximum load signage at hoist entry.
- Never allow personnel to ride material hoists unless dual-rated.
- Communicate with spotters or signalers — no unexpected movement.

👉 DEMO SUGGESTION: Walk to the site hoist and review the posted capacity. Ask the team how they’d calculate safe loading and what happens if weight limits are ignored.

## Main Bullet Points (for Foreman)

Always know and obey the rated hoist capacity.

Secure and evenly distribute all loads before lifting.

Inspect hoist for wear, frayed cables, faulty brakes.

Keep unauthorized workers off and out of the hoist area.

Never ride a material hoist unless it’s dual-rated.

Use spotters and visual/audible communication for every lift.

Shut down and report hoist malfunctions immediately.

Post max weight signs clearly — and reinforce daily.

## Engagement Tip

- Ask: “How do you know if your load is too heavy for the hoist?”
- Optional: Roleplay a miscommunication between operator and spotter and how to fix it.