# Tailgate Safety Meeting #58

# Swing Stage & Suspended Scaffold Safety (OSHA Compliant)

Duration: 10–15 minutes

Audience: All workers involved in exterior envelope work, window installation, façade repair, or high-rise painting

Objective: To educate workers on the safe setup, use, and emergency procedures for swing stages and suspended scaffolds.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Swing stages, also called suspended scaffolds, are platforms hung by ropes or cables from overhead supports. They're used for working on building facades, windows, or exteriors on high-rise structures. These systems offer efficient access to the sides of tall buildings but present serious hazards if not used properly.

The most common dangers include:
- Falls due to missing guardrails, harness failure, or overreaching
- Scaffold collapse due to overloaded platforms or faulty suspension cables
- Electrocution from overhead power lines
- Weather-related instability (wind, rain, lightning)
- Inadequate anchorage or tie-back failure

Key Safety Procedures:
- Always inspect the swing stage before each use.
- Use a personal fall arrest system (harness + lanyard + anchor point).
- Ensure independent lifelines are secured to separate anchorage points—not the same anchor as the platform.
- Keep both guardrails and toe boards in place.
- Test brakes, hoists, and descent mechanisms before use.
- Never exceed the rated weight capacity of the scaffold.
- Ensure tie-backs are installed and secured correctly to roof anchors.
- Stop work in high winds or when storms are approaching.
- Maintain 10-foot clearance from electrical lines.

👉 DEMO SUGGESTION: Show a properly assembled harness and practice connecting to a tie-back point. Walk the crew through a visual inspection of the swing stage or show photos if not available on site.

## Main Bullet Points (for Foreman)

Inspect the swing stage system daily before use.

Tie-back cables must be anchored independently of suspension lines.

Use a full-body harness and ensure proper tie-off at all times.

Keep guardrails, toe boards, and access gates secured.

Be alert for weather changes and stop work during wind or lightning.

Check electrical clearance—10 ft minimum from live wires.

Never allow tools or materials to accumulate on the platform.

Train workers on emergency lowering procedures and rescue plans.

## Engagement Tip

- Ask: “What would you do if one side of the scaffold stopped working during descent?”
- Optional: Walk through a simulated emergency response scenario or demonstrate the harness inspection checklist.