# Tailgate Safety Meeting #63

# Hoist Rigging Safety for Curtain Wall and Steel Components (OSHA Compliant)

Duration: 10–15 minutes

Audience: Ironworkers, glaziers, riggers, signal persons, and crane operators handling structural and façade lifts.

Objective: To ensure all workers involved in rigging, hoisting, and guiding large steel or curtain wall components understand proper rigging practices, load control, and fall hazard mitigation.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Lifting curtain walls and structural steel components requires strict adherence to rigging and hoist safety procedures. A mistake in rigging or load control can cause thousands of pounds to fall, putting lives at risk and damaging expensive materials.

Key Hazards:
- Load shifting or detaching midair
- Riggers in pinch zones or swing radius
- Overloaded slings or improperly rated hardware
- Failure of clamps, hooks, or suction lifters
- Wind gusts destabilizing large or flat objects

Rigging Best Practices:
- Always use rigging equipment rated for the load with proper safety factors.
- Inspect slings, shackles, and lifting points before every use.
- Tag out and replace frayed slings or cracked hooks.
- Balance loads using spreader bars or dual-pick points.
- Never walk under a suspended load—enforce this rule strictly.
- Communicate using radios or hand signals—have a designated signal person.
- Coordinate all lifts with the crane operator and foreman.
- Be mindful of wind conditions when lifting glass, panels, or frames.

👉 DEMO SUGGESTION: Show proper use of a choker hitch, basket hitch, or spreader bar. Walk through the lift plan and tag line setup with the crew.

## Main Bullet Points (for Foreman)

Inspect all rigging gear before every lift—look for wear, tears, bends, or rust.

Use appropriate rigging methods based on load size, weight, and shape.

Communicate clearly using hand signals or radios—only one signal person per lift.

Establish and maintain exclusion zones under all suspended loads.

Never use makeshift rigging—only rated, certified equipment.

Follow the lift plan exactly—no improvising once lifting has started.

Tag lines should be used to help control long or swinging loads.

Always stop work if wind, visibility, or coordination becomes unsafe.

## Engagement Tip

- Ask: “Who here has seen or been part of a near miss while rigging?”
- Optional: Walk through a real or mock rigging inspection and explain why each part matters.