# Tailgate Safety Meeting #67

# High-Rise Material Staging and Drop Zone Safety (OSHA Compliant)

Duration: 10–15 minutes

Audience: All high-rise crews including laborers, operators, riggers, and supervisors.

Objective: To prevent struck-by incidents and material drop hazards during high-rise construction by managing drop zones, staging, and securing protocols.

## Script to Read Aloud (7–10 minutes)

Foreman Reads:

Working on multi-story buildings means that what we drop from above can injure or kill someone below. Every tool, bolt, or piece of debris becomes a missile when dropped from height. We need strict drop zone controls, organized material staging, and clear communication to avoid tragic accidents.

Key Hazards:
- Struck-by injuries from falling tools or materials
- Unsecured loads swinging or falling during hoisting
- Improper staging of materials leading to trip or collapse risks
- Lack of awareness of active work zones overhead

Safety Measures:
- Establish clearly marked drop zones with barricades and signage—no entry except for authorized personnel.
- Coordinate with crane operators and signalmen before lifting or lowering materials.
- Use toeboards, netting, and tool lanyards when working on elevated platforms.
- Secure all tools when not in use. Use tool belts or tethers.
- Never store materials near edges without proper guardrails or restraints.
- Communicate when staging or lifting is taking place—radios or verbal signals.
- Conduct routine checks to clear debris from staging areas and pathways.

👉 DEMO SUGGESTION: Walk the job site and point out active drop zones. Demonstrate proper staging techniques using strapping or securing blocks. Review a tool tether or netting system.

## Main Bullet Points (for Foreman)

Mark drop zones and restrict access with barricades and signage.

Use netting and toeboards on scaffolding and platforms.

Tether tools when working above others.

Avoid staging heavy materials near unguarded edges.

Ensure hoisting zones are clear and properly communicated.

Clean staging areas of clutter and debris regularly.

Use a spotter when moving or staging materials at height.

## Engagement Tip

- Ask: “What’s the closest call you’ve seen from a dropped item on site?”
- Optional: Review a short video or simulation showing what happens when a tool is dropped from height.